OSY-E PROGRAM
CLASS B TUG PROPULSION SELECTION
LAUNCH SEQUENCE AND EARLY LOGISTICS
REV A — FOR ATB REVIEW

- PURPOSE

This document records the propulsion trade history, selection rationale, and initial deployment plan for the Class B Tug, the workhorse vehicle responsible for:

- Ferrying OSY-E modules from LEO to GEO.

- Supporting early OSY-E construction operations.

- Conducting the first pod-retrieval cycles (25–50 pods).

- Establishing the logistical backbone until OSY-E fuel production activates.

This document captures the decision-making process, defines the Phase 1–2 architecture, and provides ATB with a technical baseline for subsystem design.

- MISSION REQUIREMENTS SUMMARY

The Class B Tug must:

- Operate between GEO (station) and an elliptical orbit with perigee at 350–400 km.

- Complete full GEO → Elliptic → GEO cycles in under 7 days.

- Retrieve 25–50 pods per mission (1 t each).

- Perform early OSY-E assembly transport: ferrying modules delivered by Starship V3 from LEO to GEO.

- Operate independently before OSY-E fuel production exists.

- Support multiple cycles without excessive downtime.

- Fit within mass and volume interfaces deliverable by Starship V3.

- Use a propulsion system compatible with long-term OSY-E logistics.

These requirements are time-driven first, mass-driven second, and cost-driven third.

- PROPULSION ARCHITECTURES CONSIDERED

3.1 Electric Propulsion (EP-Only: NEP / Hall / Nested Hall)

Considered systems:

- NEP ion engines

- Hall clusters

- Nested Hall HERMeS-style setups

- VASIMR low-power configurations

Key findings:

- Thrust-to-mass ratios are too low by multiple orders of magnitude.

- Even at 1 MW electric, transit times are weeks to months, not days.

- GEO–LEO elliptical insertion requires km/s in hours, not continuous low thrust.

- Excellent for later barge-class tugs, not for Class B.

Conclusion: Rejected for primary propulsion. Retained only for stationkeeping and fine ops.

3.2 Chemical Propulsion (Methalox, Hydrolox)

We evaluated:

- LOX/CH4 vacuum engines

- LOX/LH2 vacuum engines (RL10 class analogs)

Observations:

- Hydrolox gives strong Isp (~450 s) and workable mass fractions.

- Methalox is easier to store but takes a heavy dv penalty (~360 ss Isp).

- To meet the 7-day cycle with 25–50 pods, burns must be hour-scale, not tens of hours.

- Both require significant prop loads; hydrolox is acceptable, methalox marginal.

Conclusion: Hydrolox is viable but suboptimal. Methalox marginal.
Chemical-only is feasible but not ideal for long-term scaling.

3.3 Nuclear Electric Propulsion (NEP)

Evaluated 100 kW–1 MW class NEP systems.

Observed:

- Same limitations as EP-only: thrust is too low for 7-day cycles.

- Excellent long-term for barge and cargo hauling over weeks/months.

- Cannot handle early OSY-E logistics tempo.

Conclusion: Rejected for Class B. Reserved for barge-class.

3.4 Nuclear Thermal Propulsion (NTP)

Evaluated reactor sizes, thrust levels, Isp performance.

Key data:

- Isp: 850–900 s

- Thrust: tens of kN with compact reactor assemblies

- Mass fraction drastically improves vs chemical

- Burn durations: hours rather than days

- Fully compatible with 3–3.5 km/s mission dv

- Supports 7-day cycle with margin

- Compatible with Earth-delivered LH2 until OSY-E fuel capacity comes online

- Reactor mass within Starship V3 capability

Conclusion: Only propulsion system meeting all mission constraints.

Final selection: NTP primary propulsion for Class B Tugs.

- CLASS B TUG - BASELINE DESIGN (BLOCK 0)

4.1 Mission Scale

- Pod capacity: 25 pods (25 t).

- Dry tug mass (reactor, engine, tanks, avionics, RCS, structure): 20–25 t.

- Total dry system: 45–50 t.

4.2 Propellant Load

- Required dv: 3,500 m/s after margin.

- At Isp ~900 s, mass ratio ~1.49.

- Tug departure mass ~75 t.

- LH2 propellant ~25 t per cycle.

4.3 Thrust Requirements

- Required thrust band: 20–50 kN for hour-scale burns.

- Ensures GEO insertion and de-orbit burns occur within mission windows.

- Allows rapid departure and return legs (2–3 days each).

4.4 Propulsion Stack

- Primary: NTP engine cluster (single core or dual-mode configuration).

- Secondary:

  - Chemical trim engines (storable, low thrust)

  - NH3 or H2 RCS system

  - Optional: 10–50 kW electric thrusters for fine orbit shaping

4.5 Reactor + Shielding

- Compact reactor optimized for high thrust NTP.

- Forward shadow shield to protect avionics and pods.

- Rear thermal radiators sized for safe hot soak.

4.6 Docking and Capture Hardware

- Reinforced pod mount rail for 25-pod configuration.

- Hard dock interface with OSY-E core ring.

- Bots mountable for capture assistance.

4.7 Turnaround Operations

- LH2 top-off from Starship tankers (Phase 1).

- Quick-inspection cycle < 12 hours.

- Autonomous health monitoring with worker bot integration.

- STARSHIP V3 BOOTSTRAP SEQUENCE (PHASE 1)

Goal: Deliver OSY-E Core + first tug + early power/robotics to GEO.

This architecture assumes Starship V3 delivers modules to LEO, and the Class B Tug ferries them to GEO.

5.1 Starship V3 - Flight 1
"OSY-E Core Delivery"

Delivered to LEO:

- OSY-E Core Module:

  - Central hub structure

  - Primary docking ring

  - Attitude control system

  - Comms backbone (Earth link, inter-sat, local net)

  - Initial power (small solar arrays + batteries)

  - Grapple fixtures and assembly hardpoints

  - Propulsion for LEO stationkeeping until tug arrival

Mission:

- Tug retrieves OSY-E Core from LEO.

- Ferries to GEO in first operational NTP burn series.

- Worker bots (arriving later) will mount and deploy additional components.

5.2 Starship V3 - Flight 2
"Power and Robotics Module"

Delivered to LEO:

- Phase 1 Power System:

  - Expanded solar wings

  - Additional batteries

  - Power distribution racks

- Robotics Package:

  - Assembly bots

  - Inspection drones

  - Modular tool bays

- Radiators for thermal management

- Structural expansion frames

Mission:

- Tug ferries this entire module to GEO.

- Worker bots assemble power arrays, expand OSY-E capability.

5.3 Starship V3 - Flight 3
"Tug + Cryo Depot Package"

Delivered to LEO:

- Class B Tug (Block 0), fully integrated

- Cryogenic LH2 Depot Module:

  - Multi-tank cluster

  - Lines, valves, chilldown equipment

  - Passive insulation + boil-off tolerant architecture

- Additional robotics units

- Utility racks for tug servicing

Mission:

- Tug ferries itself + depot module to GEO.

- Establishes the first operational OSY-E fuel resupply node.

- Initiates readiness for pod retrieval operations.

- EARLY LOGISTICS FLOW (PRE-FUEL PRODUCTION)

Cycle:

- Starship tankers deliver LH2 to LEO.

- Tug ferries LH2 tankage to GEO depot.

- Tug refuels directly from depot.

- Tug begins first pod cycles (25 pods → OSY-E).

- OSY-E receives early water-processing modules on later Starships.

- Once fuel production begins:

  - LH2/O2 can be made onsite

  - NTP cycles become more frequent

  - Chemical engines (hydrolox) for auxiliary tasks become practical

  - Tug 2 and Tug 3 assembly begins on-orbit

This allows OSY-E to enter its growth phase (Phase 1–4 major construction).

- WHY THIS ARCHITECTURE IS SELECTED

7.1 Time Compliance

NTP is the only propulsion system capable of:

- 7 day GEO <-> LEO elliptical round trips

- 25–50 pod retrieval

- Early module ferry duty

- High-thrust hour-scale burns

All alternatives fail the time requirement.

7.2 Logistics Chain Compatibility

The NTP architecture integrates cleanly with:

- Early Earth-supplied LH2

- Later OSY-E water splitting

- Potential hydrolox chemical tugs

- Future thermal-electric upgrades

- All station assembly operations

7.3 Scalability

The solution scales cleanly:

- Block 0: First tug, ferry/assembly duty

- Block 1: Add Tug 2 and Tug 3

- Block 2: Expand pod capacity to 50

- Block 3: Add NEP / MPD hybrid thrusters for long-haul missions

- Block 4: Support OSY-M construction and Mars logistics

7.4 Safety and Control

- Reactor stays on tug, not OSY-E

- GEO operations keep Earth biosphere safe

- NTP is operational only in vacuum far from Earth

- Worker bots minimize EVA risk

- NEXT STEPS FOR ATB

- Approve or request modifications to the Class B NTP baseline.

- Approve the 3-launch bootstrap architecture for OSY-E Phase 1.

- Begin subsystem-level ICD definition for:

  - NTP reactor and engine cluster

  - LH2 depot module

  - OSY-E docking and tug servicing architecture

  - Power and thermal interfaces

  - Pod capture hardware

- Prepare Block 1/2 development schedule.

- Initiate mass budget refinement for Starship V3 packaging.