Below is a campus-wide directive + architecture baseline you can drop straight into governance as the starting point for design, vendor specs, and MECSAI rules.

## ATLAS / SPEAR ENTERPRISE - CAMPUS DC POWER DIRECTIVE

## Document: DC-CAMPUS-ARCH-BASELINE-001

## Revision: A

## Status: ACTIVE BASELINE

## Scope: Entire campus electrical generation, storage, distribution, loads, and grid-tie export

- PURPOSE

Establish a DC-first campus power architecture with:

- A primary HVDC campus transmission backbone

- Micro-substations that step DC voltage down by conversion stages

- Building-level DC distribution with conditioning for sensitive equipment

- AC limited to grid-tie export and any explicitly approved legacy loads

- ARCHITECTURE OVERVIEW

The campus shall operate as a multi-source DC utility with segmented buses and fast DC protection.

High-level topology:

[Generation + Storage Domains]

Solar DC

Battery DC

Fuel Cell DC

Turbine/Nuclear AC -> Rectifier/DC Front End

SOEC Loads (DC)

|

v

[PRIMARY HVDC BACKBONE RING/BUS]

|

v

[MICRO-SUBSTATIONS (MVDC NODES)]

|

v

[BUILDING SERVICE ENTRANCE DC SWITCHGEAR]

|

v

[POWER CONDITIONING + POINT-OF-LOAD DC]

|

v

[SENSITIVE LOADS: Data Hall, Controls, SOEC Aux, Labs, ICS]

Grid export path only:

[HVDC Backbone] -> [Grid-Tie Inverter/Transformer] -> [Utility AC Substation]

- CAMPUS VOLTAGE DOMAINS (BASELINE)

These are baseline targets for engineering. Final selections must pass equipment availability, arc-flash, and protection studies.

3.1 Primary campus transmission backbone (HVDC)

- Baseline: 10 kV DC nominal (range 8 kV to 12 kV)

- Rationale: efficient campus distribution, manageable conductor sizing, equipment availability through MVDC ecosystems

- Topology: ring preferred with sectionalizing; radial allowed only with redundancy

3.2 Micro-substation distribution (MVDC)

- Baseline: 1500 V DC nominal (range 1000 V to 2000 V)

- Feeds: electrolyzer yards, fuel cell yards, battery yards, building clusters

3.3 Building distribution (LVDC)

- Baseline A (general building): 380 V DC for building distribution

- Baseline B (high power industrial loads): 750 V DC where needed for motor drives / high current reduction

- Baseline C (controls): 48 V DC for controls, networking, instrumentation, and safety PLC/ICS

3.4 Data center internal distribution (DC-first)

- Building service: 380 V DC distribution

- Rack/row: 48 V DC busbars or 380 V DC direct to high-efficiency PSUs, depending on rack strategy

- Any AC inside the data center requires explicit justification and approval

- POWER FLOW AND CONVERSION RULES

4.1 No uncontrolled hard-ties between major buses

- All inter-bus connections must be through bidirectional power electronic stages with fast protection and controllable ramp rates.

4.2 Required conversion stages

- HVDC backbone to MVDC node: isolated bidirectional DC-DC conversion

- MVDC node to building LVDC: isolated bidirectional DC-DC conversion

- LVDC to point-of-load: regulated DC-DC with hold-up and ride-through behavior defined per load class

4.3 Rectification policy (for turbines/nuclear)

- Turbine/nuclear generation is permitted as AC generation only if immediately rectified to the HVDC backbone via controlled active front end (AFE) rectifiers.

- Harmonic control and fault contribution must be specified and modeled.

4.4 Grid export policy

- Only the grid-tie interface converts to AC.

- Grid-tie shall be modular (N+1), black-start capable if required, and islanding compliant.

- MICRO-SUBSTATION BASELINE (MVDC NODES)

Each micro-substation shall include:

- HVDC input switchgear with sectionalizing capability

- HVDC protection: hybrid DC breaker or equivalent fast interruption technology

- Isolated DC-DC conversion blocks sized for node capacity with N+1 redundancy where required

- MVDC bus with local segmentation (multiple feeders)

- Local energy buffering as required (small BESS or supercap optional) to reduce transient stress on backbone

- MECSAI interface: telemetry + command setpoints (never direct hard real-time safety control)

- BUILDING SERVICE ENTRANCE BASELINE

Each building shall include:

- MVDC feeder input switchgear with DC-rated isolation and protection

- Building DC main bus (380 V DC or 750 V DC as designated)

- Building-level power conditioning:

  - voltage regulation

  - transient suppression

  - EMI filtering

  - hold-up / ride-through for sensitive loads

- Segmented panels: critical loads vs non-critical loads

- Local discharge/bleed-down circuits for safe servicing

- POWER QUALITY AND SENSITIVE LOAD DELIVERY

The campus shall define load classes and enforce conditioning requirements:

Class 1 - Mission critical (data hall core, safety PLC, hydrogen safety)

- Ride-through: defined by design basis (baseline 10-30 seconds via UPS DC storage or local buffer)

- Voltage regulation: tight band, fast response

- Redundancy: dual-feed from independent building buses where feasible

Class 2 - Critical process (SOEC balance of plant, compressors, pumps)

- Ride-through: short (1-5 seconds) or process-dependent

- Controlled ramp rates to avoid thermal/mechanical shock

Class 3 - General loads (HVAC, lighting, non-critical)

- Minimal conditioning; shed first during constraints

- DC PROTECTION, ARC, AND SAFETY DIRECTIVE

8.1 DC arc flash is treated as primary hazard

- Arc mitigation is mandatory at HVDC and MVDC switchgear.

- All switchgear must support arc detection (optical or equivalent) and fast interruption coordination.

8.2 Protection performance targets (baseline)

- HVDC fault clearing: single-digit milliseconds target at the local segment

- MVDC feeder clearing: low milliseconds to tens of milliseconds, depending on equipment

- Coordination must be selective: nearest device trips first

8.3 Mandatory segmentation

- The backbone must be sectionalized so no single fault can dump full campus stored energy.

- Battery yards, electrolyzer yards, and data halls must be islandable at multiple points.

8.4 Grounding and insulation monitoring

- Each voltage domain must define:

  - grounding method (solid, high resistance, or floating)

  - insulation monitoring requirements for floating systems

- Ground faults must be detected and alarmed with automatic actions per domain.

8.5 E-stops and safety PLC supremacy

- Hard real-time shutdown, purge, E-stops, and safety interlocks are handled by a safety PLC layer.

- MECSAI provides setpoints and supervisory logic only.

- MECSAI TELEMETRY AND CONTROL BASELINE

Every major node (generation, storage, micro-substation, building entrance, critical load panel) shall provide:

- V, I, P, energy flow direction

- breaker state, trip cause, fault history

- bus capacitance / stored energy estimate (or proxy)

- temperature on critical power electronics and busbars

- insulation monitor status (if floating)

- alarm summary and health score

MECSAI command set (baseline)

- enable/disable converter block

- set power limit, ramp rate, droop parameters

- request islanding / resynchronization sequence

- load shed stages (Class 3 -> Class 2 -> Class 1 only under emergency policy)

- PHYSICAL LAYOUT RULES

- HVDC corridors: dedicated raceways/trenches with clear labeling and restricted access

- Micro-substations spaced to limit feeder lengths and fault energy

- Building entry: DC gear located in fire-rated electrical rooms, arc mitigation and venting required

- Clearances: governed by selected equipment ratings and code compliance

- STANDARD INTERFACES (ICD BASELINE)

The project will maintain Interface Control Documents (ICDs) for:

- HVDC backbone to micro-substation

- Micro-substation MVDC feeders to buildings

- Building LVDC to data hall distribution

- Fuel cell DC output to MVDC node

- Battery DC blocks to MVDC node

- Turbine/nuclear AFE rectifier to HVDC backbone

- Grid-tie inverter/transformer interface to utility substation

Each ICD must specify:

- nominal voltage, operating range

- max continuous current, overload profile

- fault current contribution and clearing expectations

- connector/bus type, isolation, and grounding method

- telemetry list and command list

- APPROVAL AND CHANGE CONTROL

- This baseline is the default architecture unless an exception is approved.

- Any exception must show:

  - safety impact

  - protection coordination impact

  - efficiency impact

  - cost impact

  - MECSAI integration impact

IMMEDIATE ENGINEERING ACTIONS (DO NOW)

- Lock preliminary voltage domains: 10 kV HVDC, 1500 V MVDC, 380 V and 48 V LVDC.

- Define micro-substation standard module sizes (example: 5 MW, 10 MW, 25 MW blocks).

- Start DC protection philosophy: breaker types, clearing targets, segmentation plan.

- Build the first set of ICDs: backbone->micro-sub, micro-sub->building, building->data hall.

- Draft vendor spec language: “DC rated interruption at stated V/I, verified test evidence required.”

If you want this hardened into a NASA-style directive packet, I can format it as:

- One-page executive directive

- Full design basis (20-40 pages)

- Vendor compliance spec (procurement-ready)

- ICD templates filled for the three primary interfaces (HVDC->node, node->building, building->load)